What is Building informatics?

leveraging information technology, computer science, and related data science technologies → to improve building operation efficiency & user comfort in buildings.

dealing with collecting, managing, and analysing building data → improve building efficiency & user comfort.

What is the target of Smart Building?

user experience, increase productivity, reduce costs, mitigate physical & cybersecurity risks

What most Smart Building system contain?

HVAC, lighting, electrical, drainage & plumbing, alarm, fire safety, security & access control

What is the core of Smart Building?

Adaptability, not reactivity

What are the divers for building progression?

- Energy & efficiency

- Longevity

- Comfort & satisfaction

What is similar and difference between Smart Building and Intelligent Building?

- Similar:

  - Use information technology and data generated by building [Data]

  - Focus on creating integrated system interrelates various subsystems → single control frame work [Frame]

- Difference:

  - Smart Building also includes wider integration with utilities and city infrastructure to realize smart city （Smart Building 有公共设施和城市基建，实现 Smart city）

  - Smart Building includes use of emerging machine learning and AI for advanced control and diagnostics （Smart Building有AI和ML）

  - Smart Building encompasses the use of emerging IoT, wireless communication, and other related technologies (technologies were not developed when Intelligent building concept was proposed) （Smart Building 会用一些 Intelligent Building 时期未开发的技术）

  - Smart Building considers interaction of users with building and surrounding environment → improve comfort of users apart from building operations.

What is characteristic/feature of Smart Building?

Climate Response: buildings to adapt to external climate conditions

Grid Response: buildings to adapt the information coming from grid （电网） →  maximize the energy/economic efficiency at district/city scale

User Response: enable a real-time interaction between users and technologies → optimized performance & user comfort

Monitoring and Supervision: enable real-time monitoring of building operations and users’ behaviour.

Benefit for using Smart Building?

Lower operation cost

Lower energy cost

More flexibility

Improve user comfort, efficiency, wellbeing

Smart Building Framework? And explain the layers.

What aspect to analyse Smart building framework?

Different systems/devices within the smart buildings

Different technologies used within the smart buildings

Different use cases/features supported within the smart building

What is Smart Building underlying system?

Building automation system

What technology does BAS use?

AI, IT, IoT, Big Data, modern control…

What is the major component of BAS?

Sensor, controller, actuator, computer & server, software, network

What are BAS typical functions?

HVAC control, lighting control, Fire detection & alarm, Security & access control, Lift control, Utility management interface

What is popular Smart Building popular certification? And what is important to consider in this certification?

Smart Building collective

Consider how the building data is utilized

How many certifications in Smart Building collective? And what are they?

4, Platinum; Gold; Silver; Bronze

What themes in Smart Building collective?

- Building usage

- Building performance

- Building environment

- Health, Safety, Security

- User Behavior & collaboration

- Interactive design & Connectivity

What certification HK apply? And describe what characteristic of building?

Beam Plus, characteristic: New building, existing building, neighbourhood, interior, data centre, existing school

What are the challenges of Smart Building?

Limited interoperability – due to heterogeneity

Limited integration across system and technology

Complex dependency of different systems

Lack standard

Large scale of devices and data

Security & Privacy

What is IoT structure?

What benefit IoT have?

Save time & energy

Efficient utilisation & monitoring

Enhance real-time data use

Reduce human effort

Security & Efficiency

Get new analytic insights

Create new business opportunities

Challenges for IoT?

Heterogeneity

Integration of device

Lack of standardised protocols

Scalability

Remote device management & diagnostic

Network concern

Energy concern

Reliability concern

Privacy and Security

Solve IoT challenges?

Interoperability of devices and system

Low-cost and low-power sensing

Low-cost and high-rate networking

Cloud/edge computing

New security and privacy solution

AI ML to get new business insight

What standard does OSI model coordinate?

[ISO] standards

What Layer in OSI model use protocols and what these protocols are?

Transport layer [transmit data]

TCP & UCP

What the differences between TCP/IP and OSI model? Sketch it.

What communication medium use in IoT, is it same as protocols in IoT?

Yes, communication medium is same as protocols

Wire: BACnet, KNX, Ethernet, DALI, LonWork

Wireless: WiFi, 3G/4G/5G, Bluetooth, NBIoT, ZigBee, 6LoWPAN

What differences between wire and wireless protocols?

IoT solution for Smart Building?

Localization for occupants and resource tracking

Occupants’ safety and health security

Building health control

Resource management

Energy management

Facility management

Indoor comfort enhancement

What are Big Data Features?

5V’s

Velocity

Veracity

Variety

Volume

Value (Not all big data have it)

Big Data Challenges?

Data processing: Data mining & cleansing; Data acquiring & warehousing; Data aggregation & integration; Data analysis and modelling; Data interpretation

Data management: Cost & Operation cost; Data governess; Data and information sharing; Data ownership; Privacy & security

Great Cloud Bottleneck? And drawbacks

Bottleneck: cannot apply in IoT

Drawbacks: High cost; High response time (delay); Bandwidth congestion; Limited scalability; Privacy leakage

What does Edge computing do?

Push computation, storage, and other services close to the data sources

What is the component of Edge computing?

Local devices

Localize data centre

Regional data centre

What is Edge computing function?

Caching

Storage

Processing

Decision making

Security

What is benefit to edge computing?

Low cost

Interoperability of old and new systems

Fast response

Reliable system (intermittent Connectivity)

Security & Compliance

What is Edge network platform features?

On-demand services

Wide access to the network

Pooling resources

Rapid elastically

Measured services

What challenges of Edge network?

Heterogenetic

Resource-constraint

Dynamic network & intermittent connection failure

Sharing & management distributed network

Large scale of divices

Security